ReSpeaker 2-Mics Pi HAT v2

Complete Setup & Configuration Guide

Raspberry Pi 5 + Ubuntu

Shahil's Robot Project

Raspberry Pi 5  |  Ubuntu  |  ROS  |  TLV320AIC3X Codec

# 1. Overview & Hardware

The ReSpeaker 2-Mics Pi HAT v2 is a dual-microphone audio expansion board for Raspberry Pi. It is built around the TLV320AIC3X audio codec and provides both audio input (2 MEMS microphones) and output (3.5mm headphone jack + JST 2.0 speaker port with onboard amplifier). It connects directly to the 40-pin GPIO header.

## 1.1 Hardware Components

| Component | Details |
| --- | --- |
| Audio Codec | TLV320AIC3X (I2S interface) |
| Microphones | 2x MEMS microphones (one each side of board) |
| Audio Output 1 | 3.5mm stereo headphone jack (line-level, bypasses amp) |
| Audio Output 2 | JST 2.0 2-pin speaker port (onboard amplifier) |
| RGB LEDs | 3x APA102 (SPI controlled) |
| User Button | GPIO 17 |
| Grove Connectors | 2x Grove I2C/GPIO connectors |
| Power | Powered via 40-pin header. Micro USB port for speaker amp only |

## 1.2 Power Requirements

The HAT draws power from the Pi's 40-pin header for normal operation (codec + mics + LEDs). The Micro USB port on the HAT is only needed when driving a passive speaker from the JST 2.0 port — the onboard amplifier needs the extra current for audio playback.

NOTE: You do NOT need USB power for mic-only use. You DO need it when using the JST speaker port.

## 1.3 Output Port Differences

| Port | Type | Use Case | Needs USB Power? |
| --- | --- | --- | --- |
| 3.5mm Jack | Line-level stereo | Headphones, powered speakers | No |
| JST 2.0 (2-pin) | Amplified mono | Passive laptop/hobby speakers | Recommended |

NOTE: Speaker wire polarity (orientation of + and - on JST) does NOT affect whether you get sound — it only affects phase. Either way will produce audio.

# 2. Driver Installation (Ubuntu, Pi 5)

The standard seeed-voicecard install script does NOT work on Ubuntu — it expects Raspberry Pi OS paths like /boot/overlays which don't exist. The correct method for Ubuntu on Pi 5 is to build and install a device tree overlay manually.

## 2.1 Install Build Dependencies

sudo apt update

sudo apt install -y git make gcc device-tree-compiler i2c-tools alsa-utils

## 2.2 Build the Device Tree Overlay

git clone https://github.com/Seeed-Studio/seeed-linux-dtoverlays.git

cd seeed-linux-dtoverlays

make overlays/rpi/respeaker-2mic-v2_0-overlay.dtbo

Copy the compiled overlay to the firmware overlays directory:

sudo cp overlays/rpi/respeaker-2mic-v2_0-overlay.dtbo \

/boot/firmware/overlays/respeaker-2mic-v2_0.dtbo

## 2.3 Enable the Overlay in config.txt

echo "dtparam=i2s=on" | sudo tee -a /boot/firmware/config.txt

echo "dtoverlay=respeaker-2mic-v2_0" | sudo tee -a /boot/firmware/config.txt

Verify the lines were added:

tail -5 /boot/firmware/config.txt

## 2.4 Reboot

sudo reboot

## 2.5 Verify Detection

After reboot, check that ALSA detects the card:

arecord -l

Expected output (card number may differ):

card 0: seeed2micvoicec [seeed2micvoicec], device 0: 1f000a4000.i2s-tlv320aic3x-hifi ...

NOTE: If the make step fails due to kernel header mismatch, use the community fork instead: git clone -b v6.6 https://github.com/HinTak/seeed-voicecard/ && cd seeed-voicecard && sudo ./install.sh (replace v6.6 with your kernel version from uname -r)

# 3. ALSA Configuration

ALSA is the low-level audio layer. The TLV320 codec requires stereo output and a sample rate between 8000–96000 Hz. It does NOT support mono output — all mono audio must be upmixed to stereo via the plug plugin.

## 3.1 Hardware Capabilities (TLV320AIC3X)

| Parameter | Supported Values |
| --- | --- |
| Formats | S16_LE, S24_LE, S32_LE |
| Channels | 2 (stereo only — mono NOT supported) |
| Sample Rate | 8000 Hz to 96000 Hz |
| Card Name | seeed2micvoicec |

## 3.2 Set Default ALSA Device (~/.asoundrc)

Create this file to set the ReSpeaker as the default ALSA device and enable automatic format conversion:

cat > ~/.asoundrc << 'EOF'

pcm.!default {

type plug

slave {

pcm "hw:0,0"

channels 2

rate 48000

}

}

ctl.!default {

type hw

card 0

}

EOF

NOTE: The 'plug' type is critical — it converts mono to stereo and resamples any rate automatically. Without it you will get 'Channels count non available' errors.

## 3.3 ALSA Volume Controls

The TLV320 uses different control names than the v1 WM8960 HAT. Use numid references for reliable control:

### Playback (Output) Controls

# Main DAC volume (0-127)

amixer -c seeed2micvoicec sset 'PCM Playback Volume' 127,127

# Line output volume (JST speaker port)

amixer -c seeed2micvoicec sset 'Line DAC Playback Volume' 127,127

amixer -c seeed2micvoicec sset 'Line Playback Volume' 127,127

amixer -c seeed2micvoicec sset 'Line Playback Switch' on

# Enable DAC to Line mixer (critical — gates the signal)

amixer -c seeed2micvoicec cset numid=56 1   # Left Line Mixer DACL1

amixer -c seeed2micvoicec cset numid=60 1   # Right Line Mixer DACL1

amixer -c seeed2micvoicec cset numid=58 1   # Left Line Mixer DACR1

amixer -c seeed2micvoicec cset numid=62 1   # Right Line Mixer DACR1

# Headphone output (3.5mm jack)

amixer -c seeed2micvoicec sset 'HP DAC Playback Volume' 127,127

amixer -c seeed2micvoicec sset 'HP Playback Volume' 127,127

amixer -c seeed2micvoicec sset 'HP Playback Switch' on

amixer -c seeed2micvoicec cset numid=64 1   # Left HP Mixer DACL1

amixer -c seeed2micvoicec cset numid=70 1   # Right HP Mixer DACR1

### Capture (Microphone) Controls

# Set capture gain (numid=34, range 0-100, 60 is recommended)

amixer -c seeed2micvoicec cset numid=34 60

# Enable capture switch

amixer -c seeed2micvoicec cset numid=35 on

# Route mic inputs to PGA

amixer -c seeed2micvoicec cset numid=47 1   # Left PGA Mixer Line1L

amixer -c seeed2micvoicec cset numid=48 1   # Left PGA Mixer Line1R

amixer -c seeed2micvoicec cset numid=51 1   # Right PGA Mixer Line1R

amixer -c seeed2micvoicec cset numid=52 1   # Right PGA Mixer Line1L

### Save ALSA State

sudo alsactl store

This saves settings to /var/lib/alsa/asound.state and restores them on boot.

# 4. PipeWire / GNOME Audio Configuration

Ubuntu uses PipeWire (with PulseAudio compatibility) as the audio server. GNOME's sound panel reads from PipeWire — not ALSA directly. Even if ~/.asoundrc is configured correctly, GNOME will use its own default sink. Both layers must be configured.

## 4.1 Identify PipeWire Sinks

pactl list sinks short

On a Pi 5 with ReSpeaker and HDMI connected, you will typically see:

| Sink Name | Device |
| --- | --- |
| alsa_output.platform-soc_sound.stereo-fallback | ReSpeaker HAT (use this) |
| alsa_output.platform-107c706400.hdmi.hdmi-stereo | HDMI audio output |

## 4.2 Set Default PipeWire Sink

pactl set-default-sink alsa_output.platform-soc_sound.stereo-fallback

## 4.3 Set Volume via PipeWire

PipeWire volume uses a scale of 0–65536 for 0–100%. It supports boosting above 100%:

| Volume | pactl Value | Command |
| --- | --- | --- |
| 100% | 65536 | pactl set-sink-volume alsa_output.platform-soc_sound.stereo-fallback 65536 |
| 150% | 98304 | pactl set-sink-volume alsa_output.platform-soc_sound.stereo-fallback 98304 |
| 180% | 118014 | pactl set-sink-volume alsa_output.platform-soc_sound.stereo-fallback 118014 |
| 200% | 131072 | pactl set-sink-volume alsa_output.platform-soc_sound.stereo-fallback 131072 |

NOTE: Boosting above 100% may cause audio clipping/crackling depending on the speaker. 200% is the current setting for the robot.

## 4.4 Make Default Sink Permanent

mkdir -p ~/.config/pipewire/pipewire.conf.d

cat > ~/.config/pipewire/pipewire.conf.d/default-sink.conf << 'EOF'

context.properties = {

default.configured.audio.sink = "alsa_output.platform-soc_sound.stereo-fallback"

default.configured.audio.sink.volume = 2.0

}

EOF

# Restart PipeWire to apply

systemctl --user restart pipewire pipewire-pulse

## 4.5 Unmute

pactl set-sink-mute alsa_output.platform-soc_sound.stereo-fallback 0

# 5. Complete One-Shot Setup Script

Save this as setup_respeaker.sh and run it after every fresh OS install. It configures ALSA mixer levels, sets PipeWire defaults, and saves state.

#!/bin/bash

# ReSpeaker 2-Mics HAT v2 — Full Audio Setup Script

# Run after driver installation and reboot

CARD="seeed2micvoicec"

SINK="alsa_output.platform-soc_sound.stereo-fallback"

echo "==> Setting ALSA playback levels..."

amixer -c $CARD sset 'PCM Playback Volume' 127,127

amixer -c $CARD sset 'Line DAC Playback Volume' 127,127

amixer -c $CARD sset 'Line Playback Volume' 127,127

amixer -c $CARD sset 'Line Playback Switch' on

amixer -c $CARD cset numid=56 1

amixer -c $CARD cset numid=60 1

amixer -c $CARD cset numid=58 1

amixer -c $CARD cset numid=62 1

amixer -c $CARD sset 'HP DAC Playback Volume' 127,127

amixer -c $CARD sset 'HP Playback Volume' 127,127

amixer -c $CARD sset 'HP Playback Switch' on

amixer -c $CARD cset numid=64 1

amixer -c $CARD cset numid=70 1

echo "==> Setting ALSA capture levels..."

amixer -c $CARD cset numid=34 60

amixer -c $CARD cset numid=35 on

amixer -c $CARD cset numid=47 1

amixer -c $CARD cset numid=48 1

amixer -c $CARD cset numid=51 1

amixer -c $CARD cset numid=52 1

echo "==> Saving ALSA state..."

sudo alsactl store

echo "==> Configuring ~/.asoundrc..."

cat > ~/.asoundrc << 'EOF'

pcm.!default {

type plug

slave { pcm "hw:0,0" channels 2 rate 48000 }

}

ctl.!default { type hw card 0 }

EOF'

echo "==> Configuring PipeWire default sink..."

mkdir -p ~/.config/pipewire/pipewire.conf.d

cat > ~/.config/pipewire/pipewire.conf.d/default-sink.conf << 'EOF'

context.properties = {

default.configured.audio.sink = "$SINK"

default.configured.audio.sink.volume = 2.0

}

EOF'

systemctl --user restart pipewire pipewire-pulse

pactl set-default-sink $SINK

pactl set-sink-volume $SINK 131072

pactl set-sink-mute $SINK 0

echo "==> Done! Test with:"

echo "    aplay /usr/share/sounds/alsa/Front_Center.wav"

# 6. Testing

## 6.1 Test Playback

# Play system sound (uses default device)

aplay /usr/share/sounds/alsa/Front_Center.wav

# Play directly to ReSpeaker

aplay -D plughw:0,0 /usr/share/sounds/alsa/Front_Center.wav

# Generate 1kHz test tone (requires sox)

sudo apt install -y sox

play -n -c1 synth 3 sine 1000

## 6.2 Test Microphone Recording

# Record 5 seconds then play back

arecord -D plughw:0,0 -f S16_LE -r 16000 -d 5 -t wav test.wav && \

aplay -D plughw:0,0 test.wav

## 6.3 Check Mic Signal Level (VU Meter)

arecord -D plughw:0,0 -f S16_LE -r 16000 -vv -t wav /dev/null

Speak while watching the bars. If bars are maxed (clipping), reduce capture gain:

amixer -c seeed2micvoicec cset numid=34 40

If bars barely move, increase gain:

amixer -c seeed2micvoicec cset numid=34 80

## 6.4 Check PipeWire Routing

# Check current default sink

pactl get-default-sink

# List all active audio streams

pactl list sink-inputs short

# Move a stream to ReSpeaker (use stream ID from above)

pactl move-sink-input <STREAM_ID> alsa_output.platform-soc_sound.stereo-fallback

# 7. Troubleshooting

| Problem | Cause | Fix |
| --- | --- | --- |
| arecord -l shows no seeed card | Driver not loaded / overlay not installed | Re-check /boot/firmware/config.txt has dtoverlay=respeaker-2mic-v2_0 and reboot |
| Channels count non available | TLV320 only supports stereo, not mono | Use type plug in ~/.asoundrc with channels 2 |
| No sound from JST speaker | DAC mixer switches off or volume at 0 | Run the full setup script — especially cset numid=56/58/60/62 |
| No sound from 3.5mm jack | HP mixer switches off | Run cset numid=64 1 and cset numid=70 1, and set HP Playback Switch on |
| Sound plays but goes to HDMI | PipeWire default not set | Run pactl set-default-sink alsa_output.platform-soc_sound.stereo-fallback |
| GNOME sound panel ignores setting | GNOME overrides PipeWire default | Set in ~/.config/pipewire/pipewire.conf.d/default-sink.conf and restart PipeWire |
| Crackling / distortion | PipeWire boost too high (clipping) | Reduce pactl volume to 65536 (100%) or 98304 (150%) |
| Mic bars maxed in VU meter | Capture gain too high | amixer -c seeed2micvoicec cset numid=34 40 |
| Mic bars flat / no signal | PGA mixer switches off | Run cset numid=47/48/51/52 to route mic inputs to PGA |
| amixer: Unable to find control | Wrong control name (v1 vs v2 codec) | v2 uses TLV320 names — use numid references instead of name strings |
| seeed-voicecard install.sh fails | Script designed for Raspberry Pi OS | Use seeed-linux-dtoverlays method (Section 2) for Ubuntu |
| GPIO button not working | RPi.GPIO not supported on Pi 5 | Use gpiozero library instead: from gpiozero import DigitalInputDevice |

# 8. Quick Reference Card

## Key Commands

| Task | Command |
| --- | --- |
| List playback devices | aplay -l |
| List capture devices | arecord -l |
| Test playback | aplay /usr/share/sounds/alsa/Front_Center.wav |
| Record 5s and play back | arecord -D plughw:0,0 -f S16_LE -r 16000 -d 5 -t wav test.wav && aplay -D plughw:0,0 test.wav |
| Check mic VU level | arecord -D plughw:0,0 -f S16_LE -r 16000 -vv -t wav /dev/null |
| Open ALSA mixer GUI | alsamixer (then F6 to select card) |
| Save ALSA state | sudo alsactl store |
| Check PipeWire default | pactl get-default-sink |
| Set PipeWire volume 200% | pactl set-sink-volume alsa_output.platform-soc_sound.stereo-fallback 131072 |
| Restart PipeWire | systemctl --user restart pipewire pipewire-pulse |
| Check hardware params | aplay -D hw:0,0 --dump-hw-params /usr/share/sounds/alsa/Front_Center.wav |
| List all ALSA controls | amixer -c seeed2micvoicec controls |

## Current Configuration (Shahil's Robot)

| Setting | Value |
| --- | --- |
| ALSA Card Index | 0 (seeed2micvoicec) |
| Default Output | JST 2.0 speaker port (passive laptop speaker) |
| PipeWire Sink | alsa_output.platform-soc_sound.stereo-fallback |
| PipeWire Volume | 200% (131072) — intentional crackle for robot character |
| Sample Rate | 48000 Hz stereo (plug converts all formats) |
| Capture Gain | numid=34 = 60 (out of 100) |
| OS | Ubuntu on Raspberry Pi 5 |
| Codec | TLV320AIC3X (HAT v2) |

NOTE: The crackling at 200% volume is intentional — it gives the robot a distinctive audio character. To get clean audio, set PipeWire volume to 65536 (100%) and keep only ALSA levels at max.